MADRIL Catchment Specifications 2022

SPECIFIC TASKS WATER CATCHMENT

1.  Vertical pipe supports will be buried 36” and filled with concrete with a minimum of

3” concrete cover between ground and pipe.  2 7/8” or larger pipe shall be used.

2.  The structure is composed of two separate frames.  The outer members of each

frame shall be constructed of 4 each 6” receiver channel purlins.  All receiver

channel purlin-to-pipe and purlin-to-purlin vertical and lateral connections must

have a gusset installed.

3.  Welds shall be continuous along all mating surfaces, no tack welds.

4.  Where C-purlins tie into receiver channel purlins along outer edges of frame will

have continuous welds.

5.  Straps connecting purlins to pipe support frame are 2 ½” x ¼”.

6.  Pipe to pipe gussets are to be a minimum 6” x ¼”.

7.  Vertical-pipe-to-purlin gussets are to be minimum 6” x ¼”.  Lateral-pipe-to purlin

gussets are to be minimum 6” x ¼”.  All gussets welded to pipe shall be saddle-cut for proper weld connection.

8.  Pipe-to-pipe connections: vertical pipe shall be saddle-cut for proper welded

connection.

- Gap between frames should be no less than 6 ½” to accommodate 6” gutter (to storage).

- Frame is level front-to-back.  Gutter is sloped vs sloping frame.  Slope gutter 3” toward

storage.