MADRIL Pipe Fence Specifications 2022

- Supports:  Supports can either be driven or augured and set in cement. If driven, post shall be driven 3ft below ground.  If augured, set in cement all top rail supports/uprights (posts) at a depth of 2 feet below ground, with the exception of gate hinge post and corner posts which shall be buried 3 feet below ground level.  All supports shall be set at 40 inches above natural grade contour of ground.   Supports shall have a saddle cut in them to accommodate top rail.  Spacing between supports (posts) will be 12 feet.

- Top rail: top rail shall be welded to rail supports at a height of 40 inches above ground. Top rail shall be one continuous piece and shall not contain any step downs or irregularities.  No exceptions will be granted!

- Cable Clips: Clips shall be welded at a height of 16 inches and a height of 26 inches above natural grade contour of ground to accommodate cable.  Clips shall be welded on the outside of post.  Clips shall be 3/8” x 4”.

- Cable Installation: Cable shall be strung through clips welded on uprights at heights of 16 inches (bottom strand) and 26 inches (second strand) above ground level, and cables shall be stretched taunt with a single stand (no splices) from corner post to corner post. Cable shall be installed on the outside of exclosure.  Cable shall be in complete runs from corner post to corner post.  Two pieces of cable shall not be spliced together to form one length between corners nor shall one strand of cable be used to around the entire exclosure.  Runs shall be from corner to corner, unless otherwise noted or discussed with project coordinator or inspector.

- Gate: A 12 foot gate shall be constructed at the designated site along enclosure.   Gate shall be constructed utilizing 2 3/8” heavy wall pipe at the same spacing as the fence.   A locking mechanism/hasp shall be installed to accommodate a lock (see diagram).  Hasp shall be constructed of ¼ inch thick x 4 inches wide strap metal and a minimum of 3/8 inch diameter chain link (see diagram).   Gate shall follow same spacing and contour as fence rail and cables.

- Expansion Sleeves: Expansion sleeves shall be incorporated into the fence at each corner (see diagram).  A one foot piece of 2 7/8” heavy drill stem pipe shall be used for the expansion sleeves.  Sleeve shall have at least six inches of expansion distance once installed.   Sleeves shall be installed in the center of each straight run.  If topography causes an abrupt break in the run, more than one sleeve may be needed.

- Springs:  springs shall be installed at each corner for a minimum of one per side.  Springs shall be 12 inches in length (see diagram).   Sample springs will be available at project site visit.  Springs shall be installed opposing the turnbuckles at each corner.

- Turn Buckles:  turn buckles shall be installed at each corner for a minimum of one per side or run.  Turn buckles shall be a minimum 20 inches in length and designed to be utilized with 5/8 or ½” cable (see diagram).  Turnbuckles shall be the closed eye type only.  Sample turnbuckles will be available at project site visit.   Turnbuckles shall be installed opposing the springs at each corner.  Maintain at least half of the turnbuckle adjustment or 9-10 inches to allow for future tightening of cable.

- Cable Clamps: A minimum of two clamps per terminal end need to be installed.  All clamp nuts shall be welded upon installation.

- Cable: Cable diameter shall be a minimum of 3/8” diameter.  If used cable is utilized, cable shall be free of obvious rust or other damage which would cause the cable to become weak and brittle.

- Spacing specifications (16”, 26”, and 40”) shall be consistent at all points along fence, such as in between posts, and not just at uprights or posts.  Keep in mind that additional posts or uprights may need to be installed in order for the fence to be in compliance with contract fence spacing specification’s, particularly within drainage crossings and uneven slopes, so plan accordingly.  [This may mean that more posts/uprights may need to be installed within that 12 foot spacing between uprights.

- Water Gap:  Water gap shall be constructed of pipe and according to provided specifications.