La Caja Water Development Rain and Evaporation Calculations

Rainwater Collection

Total Boss Tank water capacity:

2 x 1,800 = 3,600 gallons

Rainwater collection area is 1,000 square feet.

Assuming 1 inch of rainfall, total water volume delivered to the two Boss Tanks is:

1,000 x 1/12 + 83.33 cubic feet.      1 cubic foot of volume is 7.48 gallons.

Therefore, 1” of rain will put

83.33 x 7.48 = 623.3 gallons of water into the two Boss tanks.

To fill both Boss Tanks it will require:

3,600/623.3 = 5.78 inches of rain

Drinker Evaporation Rates

Each Boss Tank has a built-in drinker which is 2.5ft. x 2.5ft. square or 6.25 square ft.

The Pan Evaporation rate for most of central New Mexico is between 90 and 95 inches (7.5 ft) per year(total).

Total evaporation from each drinker per year is approximately:

2.5ft x 2.5ft x 7.5 ft = 46.875 cubic ft.

Total gallons of evaporation per year for one drinker is:

46.875 cubic ft. x 7.48 gal/cubic ft. = 350.625 gallons.

Both drinkers = 701.25 gallons per year.